# M

# Problem Statement:

Finite element model of 2D Steady State Heat Conduction in a rectangular plate with spatially varying thermal conductivity and mixed boundary conditions.

### Domain:

T∞=300K, h=25Wm2 K

k=k0(1+αx+βy)

T∞=300K

T=400K	h=25Wm2K

)

∂T∂η=0

where k0=200Wm2K, α=0.1 and β=0.05 Aluminium like material.

### Governing Differential Equation:

∂∂xkx,y∂T∂x+ ∂∂ykx,y∂T∂y=0

### Developing Weak Form:

δT(∂∂xk∂T∂x+ ∂∂yk∂T∂y)dxdy=0

Taking the first term (A1) from above equation and using integration by parts, we get

A1 = δT∂∂xk∂T∂xdxdy= y1y2δTk∂T∂xx1x2dy- y1y2x1x2dδTdxk(∂T∂x)dxdy

= y1y2x1x2∂∂xδTk∂T∂xdxdy- y1y2x1x2dδTdxk∂T∂xdxdy

Similarly second term (A2) can be written as,

A2 = y1y2x1x2∂∂y(δTk∂T∂y)dxdy- y1y2x1x2dδTdyk∂T∂ydxdy

Now combining A1 and A2 terms,

y1y2x1x2∂∂xδTk∂T∂x+∂∂yδTk∂T∂ydxdy-y1y2x1x2(dδTdxk∂T∂x+dδTdyk∂T∂y)dxdy= 0

Using green’s theorem,

y1y2x1x2∂∂xδTk∂T∂x+∂∂yδTk∂T∂ydxdy= δTk∂T∂xηx+k∂T∂yηydS

Therefore, the weak form equation can be written as,

∇δTTk∇Tdxdy= δT(k∇T)ηdS

LHS represents stiffness matrix due to heat conduction through area and RHS represents heat flux through boundary edges. It’s also called load vector.

To make above weak form more general, we need to include the convection boundary condition.

k∇Tη=-h(T-T∞)

Substituting this into the boundary integral of the above weak form(RHS), we get,

δTk∇TηdS=-δThT-T∞dS

= -δThTdS+δThT∞dS

Therefore the final weak form is,

∇δTTk∇Tdxdy+δThTdS= δThT∞dS

### Elemental Equation:

We will be using isoparametric element. Therefore, field variable can be interpolated as,

T=N1T1+N2T2+N3T3=N[de]

and geometry can also be interpolated as,

x=N1x1+N2x2+N3x3=N[x]

y=N1y1+N2y2+N3y3=N[x]

Gradient of the T can be found as,

∇T=∂T∂x∂T∂y=∂N1∂x∂N2∂x∂N3∂x∂N1∂y∂N2∂y∂N3∂yde=B[de]

From weak form equation,

∇δT=Bδde

Therefore, the LHS of weak form equation becomes,

δdeTBTkBdedxdy= δde (BTk[B]dxdy)de

Where,

[Ke]= BTk[B]dxdy=BT[B]k(x,y)dxdy

[B] matrix is constant since triangular element is used. Ke is not constant but depends on the surface integral of k(x,y).

Thermal conductivity for an triangular element can be given as,

kxe,ye=k01+αx+βy

where x=Nx and y=N[y]

Also RHS becomes,

δTk∇TηdS=deTNTqηdS

where qe=NTqηdS is the load vector for the element.

Therefore the Elemental equation becomes,

Kede=[qe]

### Interpolation function in x-y coordinate system:

N1=α1+β1x+γ1y2A

N2=α2+β2x+γ2y2A

N3=α3+β3x+γ3y2A

where A= area of triangular element

α1=x2y3-x3y2

α2=-x1y3+x3y1

α3=x1y2-x2y1

β1=y2-y3

β2=y3-y1

β3=y1-y2

γ1=x3-x2

γ2=x1-x3

γ3=x2-x1

### Transforming the element to Natural Coordinate system:

ζ

(1,1)

η

(-1,-1)	(1,-1)

Interpolation function in natural coordinate system can be found be assuming

x=a0+a1ζ+a2η

y=b0+b1ζ+b2η

Now substituting x and y values and corresponding ζ and η values we get interpolation function as,

H1=1-ζ2

H2=ζ-η2

H3=1+η2

Therefore,

x=H1x1+H2x2+H3x3

y=H1y1+H2y2+H3y3

and Jacobian matrix can be written as,

J=0.5x2-x1x3-x2y2-y1y3-y2

Therefore the elemental stiffness matrix can be found as,

Ke=k0BT1+αHx+βHy|J|dζdη[B]

Where -1≤ζ≤1 and -1≤η≤1

The order of polynomial inside the surface integral will be 1, so only 1 gauss point is required which is going to be (13,-13) and weight= 2.

For every element, Ke will change depending upon the coordinates of the vertices. The integral

-11-11HJdζdη=232323|J|

### Applying convection boundary condition:

Note that convection boundary condition is applied to only the top and right boundary edges, as convection stiffness becomes zero for the all other edges. Insulated boundary condition does not contribute to the stiffness and load vector.

The boundary edges can be treated as linear element, the shape functions will be

N1=1-ζ2

N1=1+ζ2

Using the final weak form equation,

∇δTTk∇Tdxdy+δThTdS= δThT∞dS

2nd term on the LHS acts as convection stiffness. This can be converted into matrix form as

Kh=hNTNdS

After converting into natural coordinate system, we get

Kh=h[N12N1N2N1N2N22]Le2dζ

After integration we get,

Kh=hLe62112

This the convection stiffness matrix for the top and right boundary edge.

The RHS of the above weak form can be written as,

Fh=hNTT∞dS

Fh=hT∞Le211

This the load vector due to convection at the top and right edges.

### MATLAB code results:

Dividing the entire domain in 20X10 rectangular sections and dividing each section in 2 triangles through diagonal. Therefore total no. of triangular elements obtained will be 400.

Triangular mesh obtained will as shown below,

Global stiffness matrix obtained will be,

And globel load vector obtained will be,

Temperature vector obtained will be,

Temperature distribution:

Total heat transfer from the top + right boundary edge is obtained as: 3180.5 W

Maximum Temperature: 400K

Minimum Temperature: 379

### MATLAB code:

clc;

clear;

close all;

% 1. Parameters

L  = 1.0;   % Plate length (m)

H  = 0.5;   % Plate height (m)

nx = 20;   % Number of divisions in x

ny = 10;   % Number of divisions in y

k0    = 200;  % Base thermal conductivity

alpha = 0.1;  % Conductivity variation in x

beta  = 0.05;  % Conductivity variation in y

h      = 25; % Convection coefficient

T_inf  = 300; % Ambient temperature

T_left = 400;  % Left edge temperature

tol = 1e-12;

% 2. Generating Nodes

dx = L/nx;

dy = H/ny;

nodes = [];

for j = 0:ny

for i = 0:nx

x = i*dx;

y = j*dy;

nodes = [nodes; x y];

end

end

nodes

num_nodes = size(nodes,1)

% 3. Generating triangular elements

elements = [];

for j = 1:ny

for i = 1:nx

n1 = (j-1)*(nx+1) + i;

n2 = n1 + 1;

n4 = j*(nx+1) + i;

n3 = n4 + 1;

% Triangle 1

elements = [elements; n1 n2 n3];

% Triangle 2

elements = [elements; n1 n3 n4];

end

end

elements

num_elements = size(elements,1)

% 4. Plotting triangular mesh

figure

triplot(elements, nodes(:,1), nodes(:,2),'b')

axis equal

xlabel('x')

ylabel('y')

title('Triangular Mesh')

% 5. Initialize the global matrices

K = zeros(num_nodes,num_nodes);

F = zeros(num_nodes,1);

% 6. Element loops

for e = 1:num_elements

element_nodes = elements(e,:); % element nodes

% Coordinates of nodes

x1 = nodes(element_nodes(1),1);

y1 = nodes(element_nodes(1),2);

x2 = nodes(element_nodes(2),1);

y2 = nodes(element_nodes(2),2);

x3 = nodes(element_nodes(3),1);

y3 = nodes(element_nodes(3),2);

% Jacobian Matrix

dx_dxi  = (-0.5*x1) + (0.5*x2);

dy_dxi  = (-0.5*y1) + (0.5*y2);

dx_deta = (-0.5*x2) + (0.5*x3);

dy_deta = (-0.5*y2) + (0.5*y3);

J = [dx_dxi   dy_dxi;

dx_deta  dy_deta];

detJ = det(J);

invJ = inv(J);

% Shape function derivatives wrt xi and eta

dN_dxi = [-0.5; 0.5; 0.0];

dN_deta = [0.0; -0.5; 0.5];

% Derivatives wrt x and y [B]

gradN = invJ * [dN_dxi'; dN_deta'];

dN_dx = gradN(1,:);

dN_dy = gradN(2,:);

% B matrix

B = [dN_dx; dN_dy];

% Gauss Point

xi  = 1/3;

eta = -1/3;

weight = 2;

% Shape functions at Gauss point

N1 = 0.5*(1 - xi);

N2 = 0.5*(xi - eta);

N3 = 0.5*(1 + eta);

N = [N1 N2 N3];

% Coordinates at Gauss point

x_gp = N*[x1; x2; x3];

y_gp = N*[y1; y2; y3];

% Spatially variable thermal conductivity

k = k0*(1 + alpha*x_gp + beta*y_gp);

% Element stiffness matrix

Ke = (B'*k*B)*detJ*weight;

% Assembly

for i = 1:3

for j = 1:3

K(element_nodes(i), element_nodes(j)) = K(element_nodes(i), element_nodes(j)) + Ke(i,j);

end

end

end

% 7. Apply convection boundary condition

for e = 1:num_elements

element_nodes = elements(e,:);

edge_list = [1 2; 2 3; 3 1];

for edge = 1:3

local_nodes = edge_list(edge,:);

n1 = element_nodes(local_nodes(1));

n2 = element_nodes(local_nodes(2));

p1 = nodes(n1,:);

p2 = nodes(n2,:);

% TOP EDGE

if abs(p1(2)-H)<tol && abs(p2(2)-H)<tol

Le = norm(p2-p1);

Kh = h*Le/6 * [2 1; 1 2];

Fh = h*T_inf*Le/2 * [1; 1];

edge_nodes = [n1 n2];

K(edge_nodes,edge_nodes) = K(edge_nodes,edge_nodes) + Kh;

F(edge_nodes) = F(edge_nodes) + Fh;

end

% RIGHT EDGE

if abs(p1(1)-L)<tol && abs(p2(1)-L)<tol

Le = norm(p2-p1);

Kh = h*Le/6 * [2 1; 1 2];

Fh = h*T_inf*Le/2 * [1; 1];

edge_nodes = [n1 n2];

K(edge_nodes,edge_nodes) = K(edge_nodes,edge_nodes) + Kh;

F(edge_nodes) = F(edge_nodes) + Fh;

end

end

end

K

F

% 8. Apply dirichlet boundary condition

left_nodes = [];

for i = 1:num_nodes

if abs(nodes(i,1)) < tol

left_nodes = [left_nodes; i];

end

end

for i = 1:length(left_nodes)

node = left_nodes(i);

F = F - K(:,node)*T_left;

K(node,:) = 0;

K(:,node) = 0;

K(node,node) = 1;

F(node) = T_left;

end

F

% 9. Solving system of equations

T = K\F;

T

% 10. Temperature contour plot

figure

trisurf(elements, nodes(:,1), nodes(:,2), T, 'EdgeColor','none')

view(2)

axis equal

xlabel('x')

ylabel('y')

title('Temperature Distribution')

colorbar

shading interp

% 11. Post Processing and results

fprintf('Maximum Temperature = %.2f K\n',max(T));

fprintf('Minimum Temperature = %.2f K\n',min(T));

% 12. Boundary Heat Flux Calculation

Q_total = 0;

fprintf('\nBoundary Heat Flux Results:\n');

for e = 1:num_elements

element_nodes = elements(e,:);

edge_list = [1 2;

2 3;

3 1];

for edge = 1:3

local_nodes = edge_list(edge,:);

n1 = element_nodes(local_nodes(1));

n2 = element_nodes(local_nodes(2));

p1 = nodes(n1,:);

p2 = nodes(n2,:);

% Top edge convection

if abs(p1(2)-H)<tol && abs(p2(2)-H)<tol

Le = norm(p2-p1); % edge length

% 1-point Gauss quadrature

xi = 0;

weight = 2;

% Shape functions at Gauss point

N1 = 0.5*(1-xi);

N2 = 0.5*(1+xi);

Nedge = [N1 N2];

Te = [T(n1); T(n2)]; % Edge nodal temperatures

T_gp = Nedge*Te; % Temperature at Gauss point

q = h*(T_gp - T_inf); % Boundary heat flux

J_edge = Le/2; % Jacobian for line element

Qe = q*J_edge*weight; % Heat transfer through edge

Q_total = Q_total + Qe; % Total heat transfer

fprintf('Top Edge (%d-%d): q = %.4f W/m^2\n', n1,n2,q);

end

% Right convection edge

if abs(p1(1)-L)<tol && abs(p2(1)-L)<tol

Le = norm(p2-p1);

xi = 0;

weight = 2;

N1 = 0.5*(1-xi);

N2 = 0.5*(1+xi);

Nedge = [N1 N2];

Te = [T(n1); T(n2)];

T_gp = Nedge*Te;

q = h*(T_gp - T_inf);

J_edge = Le/2;

Qe = q*J_edge*weight;

Q_total = Q_total + Qe;

fprintf('Right Edge (%d-%d): q = %.4f W/m^2\n', n1,n2,q);

end

end

end

fprintf('\nTotal Convective Heat Transfer = %.4f W\n',Q_total);